## Spectral rotation.

Example function call:

blesser3(4000,'0101.wav', '0101_lo.wav', '0101_rot.wav');

The first parameter in the call to the routine blesser3 is the maximum frequency allowed in the final spectrum. This also defines the frequency at which the speech is rotated, which is half the specified maximum frequency (MaxFreq). Signal processing begins by low-pass filtering the original speech waveform at 0.95 x MaxFreq with a 6th-order elliptical filter, both forward and backwards, so as to ensure zero-phase filtering equivalent to a 12th-order filter. This low-pass filtered speech waveform is then stored under the specified name, as well as used in further processing.

Spectral rotation uses a digital version of the simple modulation technique described by Blesser (1969, 1972). In order to preserve somewhat more of the high frequency energy in the original low-pass filtered speech signal, I have rotated around 2 kHz but 1.6 kHz was used by Blesser. Because normal and spectrally-inverted signals would lead to sounds with very different long-term spectra, the speech signal is equalised with a filter (essentially high-pass) that makes the rotated signal have approximately the same long-term spectrum as the original. This equalising FIR filter is constructed on the basis of recent extensive measurements of the long-term average spectrum of speech, but ignoring the fact that the LTASS rolls off below 120 Hz and assume it is flat from 420 Hz down (Byrne et al., 1994). The equalised signal is then amplitude-modulated by a sinusoid at MaxFreq, followed by forward-backward low-pass filtering at 0.95 x MaxFreq as described above. The total rms level of the rotated signal is set equal to that of the original low-pass filtered signal. Checks for sample values that are beyond the limits allowed for .WAV files may lead to some degree of attenuation for both the low-pass and rotated waveforms.

Blesser, B. (1972) Speech perception under conditions of spectral transformation: I. Phonetic characteristics, Journal of Speech and Hearing Research 15,5-41.

Blesser, B. A. (1969). Perception of spectrally rotated speech. Unpublished Ph.D., MIT, Cambridge, MA.

Byrne, D., Dillon, H., Tran, K., Arlinger, S., Wilbraham, K., Cox, R. H., B;Hetu, R. K., J;Lui, C;Kiessling, J;Kotby, Mn;Nasser, Nha;Elkholy, , Wah;Nakanishi, Y. O., H;Powell, R;Stephens, D;Meredith, R;Sirimanna,, & T;Tavartkiladze, G. F., Gi;Westerman, S;Ludvigsen, C. (1994) An international comparison of long-term average speech spectra, J. Acoust. Soc. Am. 96,2108-2120.